# Debris Taxonomy

## Definition

Debris size: 10x5x5cm (0.1x0.05x0.05m):

- Debris objects must be at least 10cm along one axis (length, height, width), and at least 5cm along the other two axis.

- Non-debris objects do not need to conform to this minimal size (for example, small Animals).

Dynamic Debris:

- Dynamic debris are scattered objects of trash that are found in or near the street.

- Dynamic debris are objects that are light weight and can move on their own by rolling or wind.

- Can usually be driven over by the AV.

- Examples: plastic bags, paper cups, empty boxes, blankets, glass bottles, or any other waste objects

Static Debris:

- Static debris are objects of significance on or near the street.

- Static debris are objects that are heavy in weight and can not move on their own.

- Cannot be driven over by the AV.

- Examples: boulders, crates, boxes that are not empty, car tires, cinderblocks, mattress, or other large, heavy objects

## Labeling Standards

- Turn on the ground lidar toggle.

- Dynamic Debris is to be labeled everywhere.

  - debris in the road should use the debris attribute toggle.

  - debris on the sidewalk, that might interact with the AV, is labeled as a dynamic obstacle with the debris attribute toggle.

  - debris on the sidewalk, that will not interact with the AV, is labeled as a dynamic obstacle and no debris attributes.

- Static Debris is to be labeled everywhere.

  - debris in the road should use the debris attribute toggle.

  - debris on the sidewalk should be labeled as a static obstacle and no debris attributes.

    - Static debris does not move on its own and therefore if on the sidewalk, it will not interact with the AV

- Piles of debris:

  - If there is less than a 5cm gap between objects they should be grouped into one large object.

  - If there is a large gap between the objects, they should be labeled individually.

- Construction Areas:

  - Construction equipment, cones, and posts are not to labeled as debris.

  - Debris in construction areas should be labeled.

## Attributes for this Class

The following are all attributes that can be applied to an object of this class in Gulp. See the example section for examples of each attribute.  See Glossary for more definitions.

Class - primary unit of object classification

- Dynamic Obstacle - are objects that are able to freely move on their own with little or no power from a person or vehicle.

- Static Obstacle - are objects that cannot be freely moved without assistance from a person or vehicle.

The following attributes should only be applied when the debris is in the drivable area.

Debris_is_empty - Indicates whether or not a piece of debris is occupied with other objects. This attribute is applied to objects that are also containers. Object like leaves or trash bits are not occupied by other objects and are therefore false.

- False - the debris container is occupied with other objects

    - objects like sticks or rocks are to be considered as false

- Unknown - there is no indicator to know if the debris container is occupied or not

- True - the debris container is not occupied with other objects

Debris_can_drive_over - Indicates whether or not debris can be driven over.

      - False - indicates that the debris can not be driven over (usually a static obstacle)

      - Unknown - there is no indicator to know if the debris can be driven over

      - True - indicates that the debris can be driven over (usually a dynamic obstacle)

Debris_Material - Refers to the substance that objects are made with.

    - Plastic - man-made material that can range from soft to hard

    - Paper - thin sheets of wood material

    - Cardboard - thicker sheets of wood material

    - Wood - pieces of trees that have been shaped into boards like pallets

    - Stone - rocks or boulders

    - Fabric - substance that makes up clothing or blankets

    - Metal - hard metallic objects

    - Unknown - used for piles of debris that contain several materials

    - Foliage - dead plant material or tree branches

Debris_Rigidity - Refers to how flexible an object might be.

    - Hard - objects that are not flexible and should not be driven over

    - Unknown - used to indicate that the rigidity cannot be determined.

    - Soft - objects that are flexible and can be driven over

## Debris Examples

### Dynamic Debris

If balloons are not being controlled by a person, either inflated and deflated, they are considered debris.

## Attribute Examples

### Debris is Empty Attribute

Select true anytime you can confirm with certainty that the debris container in question is empty. This applies to plastic bags in the road, wrapped up blankets, and potentially to some boxes.

Select false anytime you can confirm with certainty that the debris container in question is not empty. This applies to objects that can hold objects or materials like plastic bags, trash bags, or boxes.

Select unknown anytime you are uncertain that the debris container in question cannot be identified as empty or not empty.

### Debris Can Drive Over

Select true anytime you can confirm with certainty that the debris in question can be driven over. This applies to plastic bags in the road, and small pieces of miscellaneous debris such as napkins, paper, water bottles.

Select false if you can confirm with certainty that debris in question cannot be driven over. This applies to larger debris like boulders and other larger road obstructions. Anything of even potentially significant size or weight should not be driven over.

Select unknown if you are unsure if the object can be driven over.

### Debris Material

Indicate the material of the debris in question. If multiple materials are present in the object, please select the heaviest or most prevalent material.

If there are too many materials in a pile of debris then utilize the unknown attribute. Unknown is the acceptable answer in any situation of uncertainty.

### Debris Rigidity

Rigidity is an objects flexibleness.

Soft material bend easily and does not pose a problem to the AV. Soft materials include paper, lightweight plastic, empty cardboard boxes, and blankets.

Hard materials will always pose a problem to the AV and thus is a static obstacle. However, some hard materials, like hard plastics, glass, and full cardboard boxes will be dynamic obstacles.

Unknown is always the acceptable answer for uncertainty. This could also be used for miscellaneous piles of debris where the mixing of hard and soft materials is an example of unknown debris rigidity.

## Edge Cases

### Cones

Knocked over cones are not debris, they are cones.

### Sand

Sand can be both a dynamic and static obstacle.

A small pile of sand can move around on the road with the wind and therefore dynamic.

A large pile of sand will move around a little, but will need a person or vehicle to help move the majority of the pile and therefore static.