# Road Conditions Taxonomy

## Wet Roads

## Definition

wet roads present as damp with possible areas of standing water.

## Labeling Standards

- Only label drivable areas

- Dampness on the sides of the roads in the gutters is not considered a wet road.

- Do not consider wet sidewalks as wet roads.

- Lable as unsure is there is ever any doubt.

## Attributes

Dry - The drivable area is obviously dry. The segment has no shiny, long, and narrow reflections coming from traffic lights, streetlights or headlights.

Partially Damp - The drivable area is obviously partially damp in 20-80% of the drivable area -OR-
The drivable area has shiny, long, and narrow reflections coming from traffic lights, streetlights or headlights indicating dampness AND dry patches that cover 20-80% of the drivable area.

Damp - The drivable area is obviously damp -OR-
The drivable area has shiny, long, and narrow reflections coming from traffic lights, streetlights or headlights indicating dampness AND no dry patches.

Unsure/Unknown - The condition of the drivable area is not immediately obvious, the road may be occluded. A scene marked as 'unknown' should only have one attribute. Please note that Unsure/Unknown is a desired label. It will deliver important data and inform future products.

## Wet Road Examples

### Dry

At Night - Much less reflection compared to damp

Sometimes minor reflections, but wide and less defined. Not “shiny”

Car headlights may have some reflections, but these will be much shorter / duller / more diffuse than damp roads

During the Day - no traffic light reflections

### Partially Damp

At Night - one can see that different parts of the road have different levels of reflectiveness from light sources.

During the Day - difference in color, indicates that roads are in a transition type state (going from damp to dry, or dry to damp)

### Damp

At Night - Shiny, long, narrow reflections from lights (traffic lights, street lights, headlights) on road surface

During the Day - Shiny, long, narrow reflections from light sources (traffic lights, street lights) on road surface. These will not be as bright as nighttime, but any noticeable amount typically indicates damp roads.

At Night - Shiny, long, narrow reflections from lights (traffic lights, street lights, headlights) on road surface